Forensic Tile Consultants

1987 Willowtree Court

Thousand Oaks, CA 91362

805-492-5552; 805-493-8453(FAX)

E-mail; don.tile@verizon.net

Technical Report 11-29-02

# “Rainfall Inside My House”

# The Shower Environment

Forensic Tile Consultants has performed thousands of site inspections and intrusive tests over the past several years as an Expert Witness for Construction Defect Investigations. After many years of bathroom inspections, it has become vividly clear that Residential Showers are a major source of water entry into the structure due to type of wall construction, improper construction practices and availability of construction details. While the typical Homeowner complaint that drives a Construction Defect Lawsuit tends to be roof and window leaks, a major source of water entry into the structure is located in the bathroom or bathrooms of the home. This specific area of water intrusion leads to structural damage, mold growth and health issues. While Architects and Contractors are aware of the weather issues associated with roof and window installations, very little emphasis is placed on properly constructing a shower to eliminate water leaks into the building envelope.

In 1997, Cecil Hunt, owner of ‘Hunt For Tile’, a Tile Contractor in Chula Vista, CA, performed a basic test to determine how much moisture was occurring inside the shower during a typical personal shower. He simply placed a glass inside the shower, on the receptor in the water spray pattern, and tracked the amount of time required to fill the glass with 6 inches of water. This occurred in 3 minutes. Using 12 minutes as a typical shower time, Mr. Hunt calculated that 24 inches of rain fell during that shower, which amounted to 8760 inches of “rain” in a one-year time frame. This figure has been used for several years in the industry by Tile Experts.

In an effort to justify this figure, or provide a more realistic figure, a review of the shower environment with respect to water or moisture is required. Currently, much emphasis has been placed on water conservation with reduced water flow showerheads. This is due to “The Energy Policy Act of 1992”, a Federal Law, that placed requirements on the manufacture of shower heads after January 1, 1994. This law established the National Water Efficiency Standard at 2.5 gallons per minute at a water flowing pressure of 80 PSI plus meeting the requirements of ANSI A112.18, 1M-1989, 7.4.3a for all showerheads except a safety shower showerhead.

Obviously, the water flow is going to vary with showerhead design and water pressure, plus the fact that there are probably more residential houses with water pressures around 60 PSI, than 80 PSI, and that would bring the resultant water flow down to about 2 gallons per minute.

Expert opinions vary on how long a “normal” shower lasts and how much water is actually used. The GAO (United States General Accounting Office) published a comprehensive study conducted by the American Water Works Association’s Research Foundation.  In it they report that the “Mean Daily Residential Water Use” for a shower is 11.6 gallons per person.

Therefore, a base line flow rate would simplify any analysis undertaken and give a standard by which to judge the results. For this analysis, the showerhead flow rate used in the calculations will be 2.5 gallons per minute as depicted by the National Water Efficiency Standards.

A showerhead sprays water in a constant pattern; in other words, it does not fall in a random pattern like natural rain. This fact does not lend itself to using a rain gauge to measure the water amount. The actual shower size also varies, along with the spray zone and splash effect of a moving body.

The only item that is constant and can be utilized in this analysis is the size, or footprint, of the shower unit. The analysis will compare the typical shower sizes found in residential houses. The water flow rate, calculated for a 12-minute shower, will be figured as covering the floor surface without draining away. This amount will then be added up for a one-year time frame. The following standardized units will be used:

Flow Rate	2.5 gallons per minute

Usage	12-minute shower, once per day

One Year	365 days

One Gallon	231 cubic inches

One Day Volume	30 gallons (6,930 cubic inches)

## “Rain Fall” Equivalents

| Shower Size | Square inch area | Inches per day | Inches per year |  |
| --- | --- | --- | --- | --- |
| Minimum size (plumbing code) | 1,024 | 6.8 | 2,482 |  |
| 48” x 48” | 2,304 | 3.0 | 1,098 |  |
| 42” x 42” | 1,764 | 3.9 | 1,424 |  |
| 36” x 36” | 1,296 | 5.3 | 1,935 |  |
| 28” x 60” | 1,680 | 4.1 | 1,497 |  |
| In spray pattern (per Cecil Hunt’s Testing) |  |  |  | 8,760 |

Conclusion:

Assuming the annual rain fall in Southern California in 2001 was 6” and other areas of the World receive over 200 inches of rain per year; we can compare the highest and lowest figures from the above chart (1098/2482) with those rain falls (6/200) and quickly realize that the moisture inside a shower can be from 5.5 to 414 times more “rain” than on the roof.

Note:

- If we use the 2 gallon per minute flow rate, the moisture inside the shower changes to; 4.4 to 329 times more than on the roof.

- If we use the “Median Daily Residential Use Per Capita” figure of 11.6 gallons, the moisture inside the shower changes to; 2.2 to 156 times more than on the roof

From all the studies and variables reviewed, the range of moisture in the shower environment varies from 2.2 to 414 times the annual rainfall experienced on the structure’s roof.

The calculations and conclusions shown here are strictly meant to point out the fact that we have more moisture inside a shower during normal use than on the roof. It is therefore, necessary to design and construct a shower with equal or better care than the roof of a house.

Common sense tells us that:

- Any water occurring inside the shower area must go to the drain, not into the structure.

- In order to stop mold growth, you have to stop the water from migrating into the structure.

Written by:

#### Don Halvorson, CTA, CTC, *CMRS

Forensic Tile Consultants

*Board Certified By The American Indoor Air Quality Council